Air University, Multan Campus

Department of Electrical Engineering & Computer Science

Digital Logic Design (DLD)

3-Bit Even Parity Generator and Checker

Submitted by: Abdul Rehman (2510115)

# 1. Objective

The objective of this project is to design, simulate, and analyze a 3-bit even parity generator and checker circuit using basic logic gates. The project demonstrates how parity bits are used for simple error detection during digital data transmission.

# 2. Introduction

Parity checking is one of the simplest forms of error detection used in digital systems and data communication. A parity bit is an extra bit added to a group of data bits so that the total number of 1s is either always even (even parity) or always odd (odd parity). This project focuses on even parity: a Parity Generator circuit produces a parity bit at the transmitting end, and a Parity Checker circuit verifies, at the receiving end, whether the received data still has even parity. If the checker detects odd parity in the received data, it signals that an error occurred during transmission.

# 3. Parity Generator

The 3-bit even parity generator takes three data bits (A, B, C) as input and produces one parity output bit (P), such that the total number of 1s among A, B, C, and P is always even. This is implemented using two XOR gates in cascade:

P = A XOR B XOR C

## 3.1 Truth Table — Parity Generator

| A | B | C | Parity Bit (P) |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 |
| 0 | 1 | 0 | 1 |
| 0 | 1 | 1 | 0 |
| 1 | 0 | 0 | 1 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 0 |
| 1 | 1 | 1 | 1 |

# 4. Parity Checker

At the receiving end, the parity checker takes the 3 data bits and the received parity bit (A, B, C, P) and produces an error output (E). If E = 0, the data was received correctly (even parity holds); if E = 1, an error is detected.

E = A XOR B XOR C XOR P

## 4.1 Truth Table — Parity Checker (no transmission error)

| A | B | C | P | Error (E) |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 0 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 0 |
| 1 | 1 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 0 |

# 5. Logic Gates Used

- XOR gates — used to compute the parity bit and the error-detection output

- Total of 2 XOR gates in the generator and 3 XOR gates in the checker

# 6. Tools Used

- Logisim — used to simulate and verify the logic circuit behavior against the truth table

- TinkerCAD — used to design and visualize the circuit using virtual logic gate components

- Microsoft Word / PowerPoint — used for documentation and presentation of the project

# 7. Working Principle

- Three data bits (A, B, C) are fed into the parity generator circuit.

- The generator computes the parity bit P using XOR gates and appends it to the data.

- The 4-bit data (A, B, C, P) is transmitted to the receiver.

- At the receiver, the checker circuit recomputes parity over all 4 bits.

- If the result (E) is 0, the data is accepted as error-free; if E is 1, an error is flagged.

# 8. Conclusion

This project successfully demonstrates the design and simulation of a 3-bit even parity generator and checker using XOR logic gates. The truth tables verified through Logisim simulation confirm that the circuit correctly generates parity bits and detects single-bit transmission errors. This is a foundational error-detection technique used in larger digital communication and memory systems.